Title (20/20 words): Development and Application of the PFAS IMplementor for Mass Spectrometry (PIMMS): Liquid Chromatography Ion Mobility Mass Spectrometry Non-Targeted Analysis Software

Co-Authors: Gregory Kudzin1; James N. Dodds1; Anna K. Boatman1; Kaylie I. Kirkwood-Donelson2; Jessie Chappel1; Dylan Johnson4; Adam Schiffenbauer3; Kakali Sarkar3; Payam Noroozi Farhadi3; Frederick W. Miller3; Alan K. Jarmusch2; Lisa G. Rider3; Erin S. Baker1

1University of North Carolina, Department of Chemistry, Chapel Hill, NC 27599, USA

2Immunity, Inflammation, and Disease Laboratory, Division of Intramural Research, NIEHS, NIH, Research Triangle Park, NC 27709, USA

3Environmental Autoimmunity Group, Clinical Research Branch, NIEHS, NIH, Bethesda, MD 20892, USA

4Integrative Bioinformatics Support Group, Division of Intramural Research, NIEHS, NIH, Research Triangle Park, NC 27709, USA

Introduction (119/120): Per- and polyfluoroalkyl substances (PFAS) are a diverse class of fluorinated molecules that bioaccumulate, leading to harmful health effects, including decreased immune response. PFAS are typically analysed using LC-MS, which relies on comparison to analytical standards and is limited to compounds with available standards. Non-target analysis (NTA) has emerged as an alternative, enabling the detection of PFAS without standards. While LC-MS NTA software accelerates this workflow, can produce false positives when studying complex matrices such as serum and blood. Ion Mobility Spectrometry (IMS) has gained popularity for NTA, as it can separate PFAS from the matrix. The PFAS IMplementor for Mass Spectrometry (PIMMS) was developed to automate LC-IMS-MS workflows, prioritizing suspects and facilitating identification of novel and emerging PFAS.

Methods (120/120): The PIMMS workflow was developed using data from human serum and alligator blood collected on an Agilent 6560 IM-QTOF. Analysis starts with a .csv feature list containing intensity, retention time, IMS drift time, collisional cross section (CCS), and mass-to-charge ratio (m/z). This data is imported into PIMMS, where algorithms remove endogenous features and prioritize fluorinated suspects. The workflow includes 11 steps: (1) Blank subtraction, (2) Mass defect scoring, (3) Feature merging filter, (4) Smearing filter, (5) Isotopic packet filter, (6) Adduct and post-source decay filter, (7) Standard reference filter, (8) Score filtering, (9) PFAS trendline selection, (10) Library searching, and (11) Final report generation. Cross-validation was then performed to confirm the consistency of the PIMMS workflow with manual identification methods.

Preliminary Data or Plenary Speakers Abstract (300/300): Currently, several automated PFAS analysis tools exist, but none fully utilize the separation provided by ion mobility in PFAS analysis. PFAS, being highly fluorinated, exhibit much higher densities than biological molecules, resulting in significantly smaller collision cross sections (CCS) relative to similarly weighted biological molecules when analyzed by drift. PIMMS leverages this property by integrating a machine learning algorithm that prioritizes molecules with CCS-to-mass-to-charge ratio (m/z) trends indicative of heavy fluorination. This CCS-to-m/z separation allows PIMMS to outperform software that relies solely on m/z trends, such as Kendrick mass defect analysis.

To further enhance utility, PIMMS incorporates algorithms to reduce false positives. Key among these is the smearing filter, which eliminates features indistinguishable from more intense preceding peaks. Unlike other tools that misidentify the tail of a saturated peak as a unique feature, PIMMS scans within defined mass, retention, and CCS windows to identify and remove such artifacts. Additionally, a post-source decay and adduct filter identifies artifacts and adducted complexes that could otherwise inflate false identifications or require manual corrections.

PIMMS has been rigorously benchmarked against manual identification workflows performed by trained technicians, the gold standard in PFAS analysis. In an unguided, non-targeted workflow, PIMMS achieved >88% concordance with human identifications while processing each sample in just 20 seconds. This efficiency enabled the analysis of over 156 human serum samples in 45 minutes, significantly accelerating PFAS detection without sacrificing accuracy.

PIMMS has also demonstrated its utility in novel applications, such as analyzing blood samples from alligators in the Cape Fear River. The software rapidly flagged features later confirmed as novel PFAS through independent MS/MS validation. As such, PIMMS offers a powerful, user-friendly, and customizable solution for non-targeted PFAS analysis. Distributed through GitHub for non-commercial use, the software allows laboratories to adapt and optimize the platform for their specific identification workflows.

Novel aspect (20/20): The development and validation of a novel open-source software for rapid, non-targeted PFAS analysis with accuracy comparable to manual workflows.