Title (20/20 words): PFAS IMplementor for Mass Spectrometry (PIMMS): Non-Targeted Analysis Software for Data with Liquid Chromatography, Ion Mobility, and Mass Spectrometry Separations

Co-Authors: Gregory Kudzin1; James N. Dodds1; Anna K. Boatman1; Kaylie I. Kirkwood-Donelson2; Jessie Chappel1; Dylan Johnson4; Adam Schiffenbauer3; Kakali Sarkar3; Payam Noroozi Farhadi3; Frederick W. Miller3; Alan K. Jarmusch2; Lisa G. Rider3; Erin S. Baker1

1University of North Carolina, Department of Chemistry, Chapel Hill, NC 27599, USA

2Immunity, Inflammation, and Disease Laboratory, Division of Intramural Research, NIEHS, NIH, Research Triangle Park, NC 27709, USA

3Environmental Autoimmunity Group, Clinical Research Branch, NIEHS, NIH, Bethesda, MD 20892, USA

4Integrative Bioinformatics Support Group, Division of Intramural Research, NIEHS, NIH, Research Triangle Park, NC 27709, USA

Introduction (119/120): Per- and polyfluoroalkyl substances (PFAS) are a diverse class of fluorinated molecules of great concern for human health. PFAS are typically analysed with targeted liquid chromatography-mass spectrometry (LC-MS) platforms, limited to only compounds with available standards. Non-targeted analysis (NTA) has emerged as a powerful alternative, enabling the detection of PFAS without standards. While current NTA software for LC-MS can accelerate this workflow, it often produces false positives in complex matrices such as blood. Ion mobility spectrometry (IMS) has recently gained popularity for NTA, as it can separate PFAS from endogenous molecules in the matrix. The PFAS IMplementor for Mass Spectrometry (PIMMS) was therefore developed to automate LC-IMS-MS workflows, prioritize suspects, and facilitate the identification of novel and emerging PFAS.

Methods (120/120): PIMMS is a stand-alone, open-source software designed to rapidly identify fluorinated suspects in complex matrices including serum and whole blood. The workflow starts with a .csv feature list containing intensity, retention time, IMS drift time, collisional cross section (CCS), and mass-to-charge ratio (m/z). Following this, a series of 9 user-controlled feature-filtration steps are initiated to reduce the potential for false positive identifications and eliminate ~98% of features. These steps address areas including: (1) blank subtraction, (2) retention time, m/z, intensity cutoffs, (3) smearing, (4) branching, (5) isotopic packet condensing, (6) machine learning CCS v m/z separation, (7) mass defect, (8) feature detection frequency, (9) stable isotope standard removal. Scoring steps are then initiated to aid in the identification of features.

Preliminary Data or Plenary Speakers Abstract (298/300): PIMMS was designed to accelerate the NTA workflow by prioritizing fluorinated suspects and eliminating endogenous features. PIMMS was developed and tested using data collected from human and alligator plasma and serum. Feature lists for these sample sets are often above 12,000 features per sample with often <1% of these features being identifiable PFAS after extensive manual validation. As a result, numerous filtering steps are necessary prior analysis to prioritize potential candidates, reduce false positive identifications and increase computational demand.

Currently, several automated NTA PFAS software tools exist, but none utilize the separation provided by IMS or enable the 9 filtering steps of PIMMS. For example, processing a feature file through steps 1-5, step 6 enables the use of the IMS dimension, which is extremely helpful in PFAS identification. Since PFAS are highly fluorinated compared to biological molecules, they exhibit much lower CCS values relative to m/z. The algorithm in step 6 therefore used the Baker Lab’s PFAS and lipid library containing over 1000 CCS and m/z features to find a gradient that can separate PFAS from a wide range of different lipids. This one filter alone was able to reduce the experimental feature list of human serum samples by ~90%, enabling higher confidence identifications.

PIMMS has been rigorously benchmarked against manual identification workflows performed by analysts. PIMMS achieved >88% concordance for emerging PFAS in human serum despite having differing workflows. It also had 100% concordance for stable isotope label mixes added to samples and enabled analysis in under one minute per sample. Moreover, in testing, no false positives were found. PIMMS has also demonstrated its utility in aiding novel PFAS discovery such as in analyzing blood samples from alligators, which will be showcased. As such, PIMMS offers a powerful, user-friendly, and customizable solution for PFAS NTA data workflows.

Novel aspect (18/20): The development and validation of an open-source software designed to drastically accelerate PFAS NTA workflows with LC-IMS-MS separations.